# SOIL INDICATION -– SOIL MANAGEMENT DECISIONS THE BASIS FOR EROSION AND COMPACTION CONTROL

Frielinghaus, Monika; Müller, Lothar; Klaus Seidel; Barbara Winnige

Centre for Agricultural Landscape and Landuse Research (ZALF) Müncheberg, Germany, Institute of Soil Landscape Research, e.mail: <frielinghaus@zalf.de>

## INTRODUCTION

Although soil degradation is recognised as a widespread problem, the geographic distribution and the total area of affected soil regions are not well-known. Slow, insidious changes in soil quality are less dramatic than highly publicised environmental or climatic disasters and are thus seldom recognised as a truly serious issue requiring immediate attention (OECD, 1998).

The inextricable connection of the pedosphere to the lithosphere, biosphere, atmosphere, and hydrosphere suggest an altered soil state may have noticeable long-range impacts on other systems. Measures to prevent or abate soil degradation should be developed in relation to the size of the impacted area and degree of soil damage (Frielinghaus et al., 1999).

For the purpose of optimising soil functionality, an assessment of soil-modifying processes is needed. These processes include but are not limited to wind and water erosion, atmospheric deposition, pesticide/herbicide application and other crop management practices. Soil loss, compaction and acidification as well as a change in soil structure, organic matter and chemical content may result (Acton et al., 1993) (Figure 1)

Indication of Soil Quality Changes

Soil quality function

Dimensions of quality

Data Bases

Ecological Regions

Ecological Areas

Soil Landscapes

Soil Inventories

Indication of soil quality to realize soil function

Sustainable crop production

Environmental sustainability

SOIL-MODIFYING

PROCESSES

Land Use Management

Chemical additions

Organic Matter

Acidification

Climate

Structure

Erosion

The DRIVING FORCES-PRESSURE-STATE-IMPACTS-RESPONSE framework (DPSIR framework) of the OECD (1994, 1998) uses recommendations for sustainable land use management as a foundation for assessing the type and extent of land use factors which influence the most important soil-modifying processes. In north-eastern Germany, soil erosion and compaction, the most significant soil degradation processes, have been linked to agricultural management practices. Controlling soil erosion and compaction requires recognising the indicators signalling soil state and degradation risks. The type of indicators relevant to the impact of agricultural production practices on soil quality are discussed in this paper.

MATERIAL AND METHODS

Different groups of indicators are necessary for assessing the impact of soil erosion and compaction on soil quality. The first group of indicators provide the basis for estimating the potential soil erosion and soil compaction risk. These processes are interconnected; decreases in infiltration capacity and soil structure stability from soil compaction lead to an increased potential water erosion risk. The second group assist in assessing the decreased soil quality state and the heightened risk of erosion or compaction over the long-term

A third group of indicators which more directly measure the effects of land management practices with respect to erosion or compaction is needed. Proxy indicators include erosion rills or gullies and emission zones as well as real soil cover as a factor of plant or plant residues. These indicators help assess the soil state and the potential threat to an altered soil quality for the purposes of reducing the erosion and compaction risk (Table 1).

# Table 1: indicator groups for the assessment of water erosion and soil compaction risks

| example water erosion |  |  |
| --- | --- | --- |
| indicator for | parameter | method / data base |
| state of soil / risk for water erosion | soil texture, inclination | substrate and inclination types of the Mesoscale Agricultural Mapping (1983) |
| state of soil / degradation rate | soil horizon depth of A,B,M compared with normal profiles | Soil Mapping Method KA 4 (1994) |
| pressure by land use | soil cover with plant or plant residues | expert matrix (indirectly), remote sensing (indirectly), field method (directly) |
| response control | soil cover with plant or plant residues | expert matrix (indirectly), remote sensing (indirectly), field method (directly) |
| example soil compaction |  |  |
| state of soil / risk for soil compaction | soil texture, humus content, soil moisture | substrate types of the Mesoscale Agricultural Mapping (1983), soil substrate definition in the KA4 (1994) |
| state of soil / degradation rate | bulk density | DIN method, penetrometer |
| pressure by land use | axial load, wheel contact area, contact area pressure, number of trafficked operations | computer program for assessment of land management and machine types |
| response control | plant development, reduction of loading by technical parameters change | remote sensing, change of management (computer program) |

## RESULTS AND DISCUSSION

1. Indication of the potential risk of water erosion and soil compaction

The relief classes and the soil substrate type classes of the MMK (Mesoscale Agricultural Soil Mapping 1:25.000) provide the basis for indicating water erosion risk in north-eastern Germany (Lieberoth et al., 1983). The five risk classes (low to very high) of this soil map are comparable with those of the USLE (Wischmeier at al., 1978) and the ABAG (Schwertman et al., 1990).

The results of a compression and shear strength test create the basis for the five compaction risk classes of different soil substrates. The classification was derived from measurements of root development. The dominating influential factor was the soil substrate, which had differing textures in the tests (Petelkau et al., 2000).

While soils with a higher clay concentration experienced little compaction, sandy soils with lower clay and silt concentrations were extremely condensed.

Different data bases and soil maps are available for the classification of the soils. The basis for the indication of potential soil compaction are the soil substrate type classes of the MMK (Mesoscale Agricultural Soil Mapping 1:25.000), such as the five classes of soil erodibility (low to very high). (Lieberoth et al., 1983)

Precise assessment of regional characteristics provides information about preferential erosion paths like ”thalwegs“. Other evaluation criteria are created by combining the erosion and compaction risks for the same region.

2. Indication of the land use influences for soil erosion

Landscapes with an optimal soil cover, such as forested areas, may have a lower soil erosion risk than agrarian areas. The real erosion risk is a factor of the intensity of agrarian use and the type of crop production management. A closed vegetation cover protects the soil surface from raindrop splash forces. Plant roots help retain soil particles and reduce the soil transport load. The extent of soil cover material, green plant mass or crop residues can be influenced by the farmer’s management and soil tillage practices (Frielinghaus et al., 2000a).

Degree of soil cover also lends for easy application in research analyses and receives general acceptance by land users and decision-makers as an important indicator of erosion risk. This indicator addresses the questions: 1. how much soil cover is necessary to reduce soil loss by erosion for a high-risk area or what is the ”critical value“ of minimum soil cover; 2. how much cover can be realised dependent on soil quality change by long-term erosion; 3. how much cover can be realised dependent on crop type, crop rotation, tillage and management practices in different regions. Based on this analysis, it is possible at first to analyse the rate of soil degradation by soil loss, which significantly impacts plant development (Frielinghaus et al., 1998). The next step is the analysis of land use pressure resulting from the less than minimal spatial and temporal soil cover (Frielinghaus et al., 2000 b).

The third is the possibility to determine the success of response by appropriate measures to increase the spatial and temporal soil cover.

About 2 t.ha-1 dry matter, or more than 30 and up to 50 % plant surface cover, is effective for initial soil protection efforts. The results from 10 years of continual measurements on plots (sandy soils, plots parameters:  48 m L x 9 m W; 7 % slope) show the significant correlation between soil cover and soil loss.

(Table 2).

The following five criteria play a decisive role in the rate of plant soil coverage: the time to establish soil cover after planting, distribution of soil cover material as a factor

Table 2:  Correlation between rate of cover, run-off and soil loss.

| Soil cover | Plant residues | Run-off volume | Soil loss |
| --- | --- | --- | --- |
| % | t-ha-1 dry matter | (% of rainfall amount) | % |
| 0 | 0 | 45 | 100 |
| < 20 | 0,5 | 40 | 25 |
| < 30 | 1 | 35 | 8 |
| ca.50 | 2 | < 30 | 3 |
| ca.70 | 4 | < 30 | < 1 |
| > 90 | 8 | < 30 | < 1 |

of the plant morphology and the kind of crop planted, time span of soil cover during the summer and winter, time lapse without soil cover dependent on the technical management system, type of soil tillage and seedbed preparation practices.

Typical intensive agrarian crop production practices (conventional system) involve numerous soil till operations to remove the residues of the first harvest before seeding the second crop.

The greatest erosion risk occurs after seedbed preparation, which is characterised by the lowest degree of soil surface roughness (Table 3). Crop selection, improved crop rotations and a change in soil tillage practices are established techniques for increasing the soil cover.

Table 3::Soil loss and runoff dependent on soil surface cover (selected results of a long term experimental plot station)

|  | rain | rain intensity | Green fallow |  | Corn into wheat stubble/ zero tillage |  | Corn into frozen catch crops (mulch seeding) |  | Corn after conventional plough tillage |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| soil cover |  |  | > 80 % |  | > 70 % |  | >60% |  | >30 bis < 60% |  |
| date | mm | I30: mm h-1 | Soil loss kg/ plot | runoff l/ plot. | Soil loss kg/ plot | runoff l/ plot. | Soil loss kg/ plot | runoff l/plot | Soil loss kg/plot | runoff l/plot |
| 03.05.1996 08.06.1996 29.06.1996 21.05.1997 12.-14.06.1997 23.06.1997 29.06.1997 17.07.1997 18.-20.07.1997 30.05.1998 06.06.1998 07.06.1998 08.06.1999 30.06.1999 06.07.1999 | 24 38 39 10 25 25 7 27 61 23 16 16 11 8 11 | ca. 10 70 64 1 2 8 8 14 83 10 ca. 10 ca. 10 13 15 11 | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 | 14 9 8 2 10 5 8 44 119 97 0 22 2 10 7 | 2 6 2 1 0 0 0 0 2 4 0 1 0 0 0 | 93 162 146 9 16 0 0 50 141 162 43 119 25 10 10 | 2 27 18 0 1 0 0 1 2 46 2 17 0 0 0 | 12 155 143 5 30 25 15 82 140 162 43 119 10 8 8 | 6 490 283 57 80 15 4 62 38 308 195 96 2 1 1 | 70 > 162 > 162 162 162 162 46 > 250 > 250 > 162 > 162 > 162 100 100 100 |

An expert system may be used to evaluate each crop as well as traditional and soil-retaining crop rotations (Frielinghaus et al., 2000b)

3. Indication of the land use influences for soil compaction

The heavy machine and transport vehicle traffic exert high specific ground pressures from high axial loads. The following traffic parameters resulted in an altered bulk density of the trafficked soils:

 wheels may produce high contact pressure per area on the soil surface, resulting in deeply routed lanes and a wide lane profile;

 high axial loads increase the compaction of the subsoil layer with top- and subsoil damage,

 high frequency traffic results in an increased soil bulk density in the lanes and intensifies the effect on the deep soil layers as well; top- and subsoil compaction increase;

 substantial soil structural damage results from wheel slip in lanes with high water content (this parameter is not quantitatively available by the indication concept).

The specifics of an individual machine, however, do not provide sufficient information. The total pressure from the sum of the axial loads and the pressured area increase successively with higher traffic frequency.

Three different components must be evaluated: the total axial loads, the respective contact pressures for each crop and the number of wheelings for each process (e.g. from soil tillage, planting, harvest) and the total number for crop rotation practices.

Table 4 shows the tolerable maximum bearing capacity as determined by the measured bulk density for each compaction risk class dependent on available water content (AWC) (Petelkau et al., 2000). A higher contact pressure than the bearing capacity leads to decreased plant development and yields (Figure 2)

Fig. 2: Correlation between the biomasses of different crops (relative) and the change of bulk densities (relative)

The comparison of the contact pressure with the maximum bearing capacity shows the impossibility of completely avoiding the risk of soil compaction in sustainable agricultural land use production. (Table 5). However, this evaluation could provide a basis for reducing the load impact.

Table 4. Tolerable contact pressure (maximal loading capacity) for the different soil  
             compaction risk classes

| Compaction risk lasses | optimal density range Level 1 |  | slightly compacted range Level 2 |  | moderately compacted range Level 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | bulk density [g/cm3] | maximum bearing capacity [kPa] | bulk density [g/cm3] | maximum bearing capacity [kPa] | bulk density [g/cm3] | maximum bearing capacity [kPa] |  |
| # AWC 50 % |  |  |  |  |  |  |  |
| Low | 1.32 | 288 | 1.37 | 371 | 1.42 | 480 |  |
| Moderate | 1.44 | 154 | 1.49 | 212 | 1.44 | 292 |  |
| ImportantSignifcant | 1.49 | 100 | 1.54 | 153 | 1.59 | 233 |  |
| High | 1.51 | 53 | 1.56 | 90 | 1.61 | 155 |  |
| very high | 1.51 | 45 | 1.56 | 88 | 1.61 | 172 |  |
| # AWC 65 - 95 % |  |  |  |  |  |  |  |
| Low | 1.32 | 255 | 1.37 | 327 | 1.42 | 420 |  |
| Moderate | 1.44 | 128 | 1.49 | 175 | 1.44 | 240 |  |
| SignificantImportant | 1.49 | 56 | 1.54 | 90 | 1.59 | 145 |  |
| High | 1.51 | 17 | 1.56 | 34 | 1.61 | 67 |  |
| very high | 1.51 | 8 | 1.56 | 20 | 1.61 | 49 |  |

Table 5: Contact pressure represented for common production methods in Germany

| Crop system | Machine width of the main system [m] | Trafficked area (multiple wheeled) [%] | Portion of total area under multiple wheeled?moderate pressure (<150 kPa) [%] | Portion of total area under multiple medium pressured (150 – 200 kPa) [%] | Portion of total area under multiple heavy pressure (>200 kPa) [%] |
| --- | --- | --- | --- | --- | --- |
| Potatoes | 4.50 | 98 | 15 | 29 | 56 |
| Sugar beets | 2.70 | 89 | 0 | 53 | 47 |
| Corn/maize | 5.00 | 66 | 12 | 22 | 66 |
| Winter wheat | 5.00 | 61 | 19 | 34 | 47 |

4. Impacts on decision-making for agricultural management practices in areas with erosion and compaction risk

The results of the assessment may determine the response level required: in a balanced situation (Level 1), “Best Management Practices” may help ensure sustainability is maintained; slightly disproportional (Level 2) results suggest additional special agricultural management techniques may be needed, while significant differences (Level 3) may indicate the need for additional land use management adjustments or changes in technical management.

- Measures to realise the Best Management Practice (Level 1):

Selection of cover crops, planting of catch crops, underseeds, site-dependent soil tillage and cultivation, changing plant production management, set aside and rotation of fallows

- Conservation tillage and special tillage (Level 2)

Design of crop rotation, conservation tillage , mulch seeding without seedbed preparation, direct seeding/ zero tillage, contour farming, rough field surface strips

- Infrastructure retarding erosion by wind and water and soil compaction (Level 3)

Arrangement/ rearrangement of the lie of the land/fields, alteration of land use type, construction of farm roads and country roads, filtering edges, shelter-belts and wind-breaks amid endangered fields and at waters

DISCUSSION

At the present, the degree of efficiency and acceptance for erosion and compaction control management is not high and therefore can not guarantee sustainable land use and soil functionality.  The best method for increasing acceptance is a regional soil indicator system combined with an environmental indicator system (McRae et al., 2000). Like the concept of “critical load inputs” for chemical pollutants, this system would make it possible to acknowledge the driving forces leading to a soil quality changes, quantify the soil state and the change of soil condition/soil quality, and assess land use practices based on soil quality  impacts. This step-by-step concept is the basis for decisions concerning the soil carrying or load capacity (Figure 3)

Indirect

Water pullutionpollution

Loss of biodiversity

Water stress

Secondary Protection

Reform of agricultural programms Specific regulations or directives

Soil loss

Soil erosion Soil function loss

Soil compaction

Emission to air,  water and soil

Soil loss

Urban expansion  (soil sealing)

Fig.3: The DPSIR Framework applied to soil

Driving forces

State

Pressure

Impacts

Response

Land development Agricultural production Natural events Water stress

Soil Degradation Nutrient and humus losses Physical deterioration Biological deterioration

Direct

CChanges in soil quality functions

Primary Protection

Development of a national / regional soil protection policy

These results may determine the response level required: In a balanced situation, Best Management Practices may help ensure sustainability is maintained, slightly disproportional results suggest additional special agricultural management techniques may be needed, while significant differences may indicate the need for additional land use adjustments or changes in technical management. The indicator system is ideal for application in north-eastern Germany, fmoraine areas in general and areas at risk to water and wind erosion and soil compaction. Further discussions concerning the realisation of soil preservation measures may help determine the degree to which a soil indication system should be supported by the agricultural policies of the federal government or the European Community.

REFERENCES:

Acton, D.F. (editor) (1993): A program to assess and monitor soil quality in Canada.

Research Branch Agriculture Canada, Ottawa.  CLBRR Contribution No. 93-49

Frielinghaus, M.; Bork, H.-R.; Kocmit, A.; Schmidt, R. (1998). Characterization of the soil changes by erosion in the German - Polish moraine region n°: 468

Symposium n°: 31, CD, ISSS Conference, Montpellier, France 1998

Frielinghaus, M.; Petelkau, H.; Deumlich, D.; Funk, R.; Müller, L.; Winnige, B., (1999). Soil Indicator System to Minimize the Risk of Soil Degradation in Northeastern Germany. 10. ISCO Conference West-Lafayette, USA (in print)

Frielinghaus, M; Bork, H.-R. (2000 a): Soil and Water Conservation in Former East Germany. IN: Napier; T.L.; Napier, S.M., Tvrdon, J. (Edit.) Soil and Water Conservation Policies and Programs. CRC Press, Boca Raton London; New York, Washington, D.C. ISBN: 0-8493-0005-3, p.343-362

Frielinghaus, M.; Winnige, B.; Schäfer, H.; Brunotte, J.(2000 b): The use of an indicator system for crop residue management and soil erosion control. 15th Conference of the ISTRO, Fort Worth, Texas, USA, CD

Karlen et al. (1997): Soil Quality: A Concept, Definition and Framework for Evaluation. Soil Sci. Soc. Am.J. 61, p. 4-10

Lieberoth, I.; Dunkelgod, P.; Gunia, W.; Thiere, J., (1983): Auswertungsrichtlinie MMK Stand 1983. AdL. FZB Müncheberg. 55 S
McRae, T.; Smith, C.A.S.; Gregorich, L.J. (editors) (2000): Environmental Sustainability of Canadian Agriculture. Ottawa, 2000; Catalig No. A22-201/2000E AAFC No.2022/E

OECD (Organization for Economic Cooperation and Development), (1994): In: Walz, R. et al. 1996: Weiterentwicklung von Indikatorensystemen für die Umweltberichterstattung Forschungsvorhaben 101 05 016 des Umweltbundesamtes, Abschlußbericht, Frauenhofer- Institut für Systemtechnik und Innovationsforschung (Fh-ISI), Karlsruhe.

OECD (Organization for Economic Cooperation and Development), (1998): Workshop on Agri-Environmental Indicators. Breakout session Group 1: Agricultural Soil Quality. COM/AGR/CA/ENV/EPOC(98)77

Petelkau, H.; Frielinghaus, M.; Seidel, K. ( 2000): A concept for assessment of soil compaction risk and options for soil compaction control. 15th Conference of the ISTRO, Fort Worth, Texas, USA, CD

Schwertmann, U.; Vogl, W.; Kainz, M., (1990): Bodenerosion durch Wasser: Vorhersage des Abtrags und Bewertung von Gegenmaßnahmen. 2. Aufl. Ulmer. Stuttgart. 64 S.

Wischmeier, W.H.; Smith, D.D., (1978): Predicting rainfall erosion losses - a guide to conservation planning. U.S. Department of Agiculture, Aricultural Handbook 537, 58 S.